Bill of Quantity: Geotechnical Investigation

| S.No | Item Description |  | Unit | Quantity | Rate | Amount | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Geotechnical Investigation |  |  |  |  |  |  |
| 1.1 | Borehole |  |  |  |  |  |  |
| 1.1 a | Mobilization of necessary plant, equipment, men and materials for the complete scope of geotechnical investigation work as per specifications, drawings and instructions of the Engineer and to complete the same within the stipulated time schedule and demobilization from the site after completion of works |  | Lump sum | 1 | 350000 | 350000 |  |
| 1.1 b | Making minimum 100mm nominal diameter boreholes at various locations (as per location shown in drawing given in Annexure B (No. D0458801-C-DE-00-CS-BHL-001)) of structures in all types of soils by suitable approved method of boring including chiseling, cleaning, providing casing pipe as required or as directed, conducting Standard Penetration Test at starting depth 1.0m from NGL and continue at every 1.5m intervals or at change of strata whichever is less and collection of disturbed soil samples, collection of ground water and collection of undisturbed soil samples at every 3.0m intervals or at change of strata whichever is less, including transportation of all the collected samples to the laboratory and back filling of bore holes on completion of the same and all works as per specifications given in document number-: D0458801-C-DE-00-CS-TSP-001 and as directed by the engineer- in-charge. |  |  |  |  |  |  |
|  | Termination Criteria (Refer section 9.1.5 in Specification Document No. D0458801-C-DE-00-CS-TSP-001) |  |  |  |  |  |  |
|  | Soil | 20 m |  |  |  |  |  |
|  | Number of bore holes |  | Nos | 11 | 16000 | 176000 | as per site condition |
|  | Weathered Rock | 10 m (depth below weathered rock level) | Nos | QRO | 3500 |  |  |
|  | Hard Rock | 5 m (depth below Hard rock level) | Nos | QRO | 3500 |  |  |
| 1.2 | Laboratory tests on soil (Lab tests shall be conducted on samples collected at specified depths as per table given in section 10.2 of Technical specifications) |  |  |  |  |  |  |
| 1.2 a | Permeability Test on soil sample |  | Nos | 11 | 5000 | 55000 |  |
| 1.2 b | Proctor Compaction test for Maximum dry density and Optimum moisture content |  | Nos | 11 | 1500 | 16500 |  |
| 1.2 c | Bulk density and natural moisture content |  | Nos | 44 | included |  |  |
| 1.2 d | Specific Gravity |  | Nos | 22 | inclued |  |  |
| 1.2 e | Sieve /Hydrometer analysis for Soil classification |  | Nos | 143 | included |  |  |
| 1.2 f | Free swell Index |  | Nos | 55 | included |  |  |
| 1.2 g | Liquid limit & plastic limit |  | Nos | 44 | included |  |  |
| 1.2 h | Unconfined compression test |  | Nos | 22 | included |  |  |
| 1.2 i | Direct Shear/Triaxial shear Test |  | Nos | 33 | included |  |  |
| 1.2 j | Consolidation Test |  | Nos | 33 | included |  |  |
| 1.2 k | Chemical analysis: Conducting chemical test to determine pH value, chloride, sulphates & sulphites and organic matter contents on |  |  |  |  |  |  |
| 1.2 l | i) Soil |  | Nos | 22 | 1000 1000 | 22000 22000 |  |
|  | ii) Water |  | Nos | 22 |  |  |  |
| 1.2 m | California bearing ratio |  | Nos | 05 | 3500 | 17500 |  |
| 1.3 | Field Tests |  |  |  |  |  |  |
| 1.3 a | Electrical resistivity test at various locations as shown in the drawing and technical specifications |  | Nos | 09 | 5000 | 45000 |  |
| 1.3 b | Excavate test pit for a depth up to 1.5m below ground level, including necessary timbering dewatering, and backfilling after excavation. Detail geological logging of pit face and base including provision for photograph |  | Nos | 05 | 6000 | 30000 |  |
| 1.3 c | Field density as per technical specifications |  | Nos | 02 | 2000 | 4000 |  |
| 1.3 e | Conducting field permeability test (FPT) as per IS 5529 (type of test shall be as per site conditions) at specified locations of Borehole 11, test starting at 1.0m below natural ground level and thereon at every 5m spacing up to a depth of 20m all complete as per specification, drawings and as directed by the engineer-in- charge. Cost also shall include of the borehole and backfilling for specified depth etc. |  | Nos | 01 | 40000 | 40000 | In Borehole 11 |
| 1.3 f | Conducting Cross-hole seismic test as per ASTM D4428/D4428M to evaluate the seismic soil properties of in-situ soil like Coefficient Shear wave velocity, Dynamic shear Modulus, Young’s Modulus |  | Nos | 1 | 80000 | 80000 |  |
| 1.4 | Tests on Rocks (Number of tests may vary based on core recovery of rock) |  |  |  |  |  |  |
| 1.4 a | Unconfined compression test (both at saturated and in–situ water content) (if applicable) |  | Nos | QRO | included |  |  |
| 1.4 b | Rock Classification, RQD, Core Recovery, Specific gravity, Hardness |  | Nos | QRO | included |  |  |
| 1.4 c | Point Load index test |  | Nos | QRO | included |  |  |
| 1.5 | Investigation report |  |  |  |  |  |  |
| 1.5 a | Preparation and submission of comprehensive soil report as per technical specifications given in D0458801-C-DE-00-CS-TSP-001 |  | Lump sum | 1 | 20000 | 20000 |  |

Additional to item no. 1.3 (f),  two boreholes down to 20m have to be drilled, shall be paid as per rate given in item no. 1.1 (b)

2BH-20m = 40m @ Rs. 800/= per meter 		::	Rs. 32,000/=

--------------------------------------------------------------------------

Total :: Rs. 9,10,000/=

Note :

Considering the subsoil conditions and earlier investigations, conducting few boreholes down to deeper depths i.e 30m  below the existing ground level is advisable.